I2C Testing code:-

I2C Init & read and write function:-

static const struct device *i2c_dev = NULL;

static bool i2c_initialized = false;

static int I2C_init(void){

if (i2c_initialized && i2c_dev != NULL) {

return 0;  // Already initialized    }

i2c_dev = DEVICE_DT_GET(DT_NODELABEL(i2c30));

if (i2c_dev == NULL) {

printk("ERROR: I2C device is NULL\n");

return -1;    }

if (!device_is_ready(i2c_dev)) {

printk("ERROR: I2C device not ready\n");

return -1;    }

uint32_t i2c_cfg = I2C_SPEED_SET(I2C_SPEED_STANDARD) | I2C_MODE_CONTROLLER;

int ret = i2c_configure(i2c_dev, i2c_cfg);

if (ret != 0) {

printk("ERROR: Failed to configure I2C peripheral: %d\n", ret);

return -1;    }

i2c_initialized = true;

printk("SUCCESS: I2C30 initialized at 100kHz\n");

return 0;

}

int32_t HAL_I2C_Write_read(uint32_t addr, uint8_t *write_data, uint32_t write_size, uint8_t *read_data, uint32_t read_size)

{

int32_t err = I2C_init();

if (err != 0) {

printk("ERROR: I2C init failed in write_read\n");

return err;        }

err = i2c_write_read(i2c_dev, addr, write_data, write_size, read_data, read_size);

if (err != 0) {

printk("ERROR: I2C write_read failed: %d (addr=0x%02X)\n", err, addr);

return err;    }

return HAL_OK; }

Sensata sensor serial number read function:-

reg_addr = 0X06;

PTE7300_SENSATA_ADDR_NO_CRC = 0X6C;

uint16_t Sensata_Read_Register(uint8_t reg_addr)

{

uint8_t read_buf[2] = {0};

int32_t status;

uint16_t value = 0;

status = HAL_I2C_Write_read(PTE7300_SENSATA_ADDR_NO_CRC, &reg_addr, 1, read_buf, 2);

if (status != HAL_OK)

{

return 0xFFFF; // error indicator

}

value = ((read_buf[1] << 8) | read_buf[0]);

return value;

}