- Firmware development GCC

- Hardware development nRF52 series DK custom shield utilizing ICSP and USBA to micro USB onto custom hardeware. Segger development

- Invensense MPU 9250 motion driver v6.12

  - TWI

  - SPI

- NordicSemiconductor’s SDK 15.0

- Copied the SDK 15.0 peripheral project TWIsensor and renamed it MPUsimpleTWI

- Hacked the main.c and the associated functions to read and write the MPU9250

  - boards.h file to point to my board file puck.h

    - puck.h was edited from the PCA10040.h

      - the clock source was the low frequency on board RC

      - associated pins especially the SDA and SCL reflected my hardware

  - Created both an i2c_read and an i2c_write function associated with the Invensene application note 3 for porting motion driver 6.12 to another MCU.

  - This was developed until reading the MPU register 0x75 WHOAMI returned 0x71 and could write to theregister 0x1F  WOM_THR and read back the value written.

    - Development is associated with the segger portion of any of the nRF52 DK